CHAPTER  1

INTRODUCTION

Today embedded systems are inventions that have made the world to think and bring  out  different  innovations  and  developments  in  the  field  of  automobiles. Embedded System is basically a system of hardware and software designed for control and access of data. As a system it includes a controller as the brain. This controller chip can be  MSP430. Today embedded systems applications are popular and their development  has become a boon to automobile industry.  Applications  in the form of home appliances, automobiles, entertainments, consumer  electronics, medical and telecommunication etc., are every day in the mode of development. Mechanical systems in automobiles are largely replaced by electronic  systems.  Today Automobile industry is making great use of embedded systems.  Ranging  from wiper  controls to  complex  anti-lock brake controls and air bags, embedded  systems have gained the overall control of  recent automobiles. The  automobiles  that are built around  using  MSP430 controllers, digital signal processors   or using both the processors are commonly called as Electronic  Control  Units. Some of  the   current trends of embedded  systems  in  automobiles include  airbag controllers, navigation systems, satellite radio, adaptive cruise control, drive by wire, heads up displays etc.

With rapidly changing computer  and  information technology and much of the technology  finding way into vehicles. They are undergoing dramatic changes in  their  capabilities  and  how  they  interact  with  the  drivers.  Although  some vehicles have provisions for deciding to either generate warnings for the human driver  or controlling   the  vehicle  autonomously, they  usually  must  make these decisions in real time with only incomplete information. So, it is important that human  drivers  still  have  some  control  over  the  vehicle.  Advanced  in-vehicle information   systems  provide   vehicles   with   different   types  and   levels   of intelligence  to  assist  the  driver.  The  introduction  into  the  vehicle  design  has allowed  an  almost  symbiotic  relationship  between  the  driver  and  vehicle  by providing  a   sophisticated  and  intelligent  driver-vehicle   interface  through  an intelligent  information  network.  This  project  aims  to  develop  such  a  control framework  for  the  vehicle  which  is  called  as  digital-driving  behavior,  consists  of  a  joint  mechanism  between  the  driver  and  vehicle  for  perception, decision making and control.

A vehicle was generally built with an analog driver vehicle  interface  for indicating  various  parameters  of  vehicle  status  like  temperature,  pressure  and speed etc. To improve the driver   vehicle interface, an  interactive digital system is designed. A MSP430  controller  based  data  acquisition  system  that  uses ADC to bring  all  control  data  from  analog  to digital format is used. Since the in-vehicle information systems are spread out all over the body of a practical  vehicle,  a communication  module  that  supports  to  implement  a  step  control  of  the  vehicle  through the  master controller of the digital driving system.

  - MSP430G2553:

The MSP430 is a mixed-signal microcontroller family from Texas Instruments. Built around a 16-bit CPU, the MSP430 is designed for low cost and specifically, low power consumption embedded applications.

The MSP430 can be used for low powered embedded devices. The current drawn in idle mode can be less than 1 µA. The top CPU speed is 25 MHz. It can be throttled back for lower power consumption. The MSP430 also uses six different low-power modes, which can disable unneeded clocks and CPU. Additionally, the MSP430 is capable of wake-up times below 1 µs, allowing the microcontroller to stay in sleep mode longer, minimizing its average current consumption.

The device comes in a variety of configurations featuring the usual peripherals: internal oscillator, timer including PWM, watchdog, USART, SPI, I²C, 10/12/14/16/24-bit ADCs, and brownout reset circuitry. Some less usual peripheral options include comparators (that can be used with the timers to do simple ADC), on-chip op-amps for signal conditioning, 12-bit DAC, LCD driver, hardware multiplier, USB, and DMA for ADC results. Apart from some older EPROM (MSP430E3xx) and high volume mask ROM (MSP430Cxxx) versions, all of the devices are in-system programmable via JTAG (full four-wire or Spy-Bi-Wire) or a built in bootstrap loader (BSL) using UART such as RS232, or USB on devices with USB support.

However, limitations that preclude its use in more complex embedded systems. The MSP430 does not have an external memory bus, so it is limited to on-chip memory (up to 512 KB flash memory and 66 KB RAM) which may be too small for applications that require large buffers or data tables. Also, although it has a DMA controller, it is very difficult to use it to move data off the chip due to a lack of a DMA output strobe.

CHAPTER 2

LITERATURE REVIEW

2.1 ANALYSIS ON EXISTING METHOD

Pradhan Suvendu Kedareswar (2015) proposed to take decision and automatically control the motion of the following vehicle, a collision avoidance system consisting of Cortex MO microcontroller is implemented that warns the drier  using  a  buzzer  and  messages  the  active  and passive safety mechanisms to  be  activated  using  CAN  protocol  and takes control  decisions according  to an algorithm designed to handle the situation.

Vijayalakshmi.S  (2013)  proposed the  communication  module  used  in this  project  is  embedded  networking by CAN which has efficient data transfer .It also takes feedback of vehicle conditions like Vehicle speed, Engine temperature etc., and controlled by main controller.

Theophilus Wellem (2010) proposed in this paper, we designed and implemented a microcontroller-based room   temperature monitoring system using Atmel ATmega8535 microcontroller and National Semiconductor’s LM35 temperature sensor.

William Prodanov (2009) proposed a behavioral model of a CAN bus transceiver is proposed and experimentally verified. Moreover, we developed a methodology to efficiently manage the trade-off concerning accuracy,  simulation  speed, and  convergence  issues which  are  usually  involved  in  the  simulation  of  large  CAN  bus networks.

Alberto Sangiovanni-Vincentelli  (2007)  described about  to optimize the system design  and  allow  for  plug  and  play of subsystems, automotive electronic system  architecture  evaluation  and  development must be  supported with a robust design flow based on virtual platforms.

Tindell.K  (2006)  proposed, One of the perceived drawbacks to CAN has been the inability to bound accurately the worst-case response time of a given message (i.e. the longest time between queuing the message and the message arriving  at  the  destination  processors). This paper presents analysis to bound such response times, including the costs of error handling and re-transmission.

Nicolas Navet (2005) described about the specific requirements of the different car domains have led to the development of a large number of auto-motive  networks  such  as  Local  Interconnect  Network, J1850, CAN,TTP/C, Flex Ray, media-oriented  system  transport, IDB1394, etc. This paper first introduces the context of in-vehicle embedded systems   and the requirements imposed on the communication systems. Then, a comprehensive   review of the most widely used automotive networks, as well as the emerging ones, is given. Next, the current efforts of the automotive industry on middleware technologies, which may be of great help in mastering the heterogeneity, are reviewed.

Keith Pazul (1999) proposed systematic approach enables interoperability between products from different manufacturers. A standard was created by the International Standards Organization (ISO) as a term-plate to follow for this layered approach. It is called the ISO Open Systems Interconnection (OSI) Network Layering Reference Model.

Cook.J.A (1993) proposed a CAN message may contain up to 8 bytes of data. A message identifier describes the data content and is used by receiving nodes to determine the destination on the network. Bitrates up to 1Mbit/s are possible in short networks (1≤40 m). Longer network distances reduce the available bit rate (125kbit/s at 500 m).  “High speed” CAN is considered to be 500kbit/s.

John  Wiley & sons  (2003) proposed  the  design process of digital automatic  control  system  with  PID (Proportional Integral Derivative)  controller is  considered. The  solution  of  problems  related  with  implementation  of   PID control  algorithm  into  general  purpose  8-bit (1byte)  microcontroller is discussed. Simulation results demonstrating performance of system.

CHAPTER 3

SYSTEM ANALYSIS

3.1 EXISTING METHOD

This system is designed to test automotive CAN control devices or entire network.  Users  can  configure  several   monitoring  modes  and  CAN  channel features of  the  system  with  PC  application.  Further more  the  USB  technology adopted in the system make it a more practical and convenient CAN bus testing system.The MCP2551  is used as the main controller of USB-CAN smart card. Its 8-bit data bus interface PORT A is used  to  communicate with PDIUSBD12 as shown in Figure 3.1. It consists of so many peripherals such as  power  module,  peripheral  electric  circuit,  USB  interface module and CAN interface module. Without the help of  PC, it  cannot   monitor the vehicle and it does not work. It requires a separate CAN transceiver for each and every nodes and separate power module is required.

In this project by using a PIC based microcontroller that is interfaced via different controllers via CAN Protocol. And also by using different analog sensors that will continuously monitor the vehicle and sending the values to the microcontroller via ADC interface. There are three analog sensors. Analog sensor used is LM35 that is used to monitor the temperature of the vehicle. And MQ-3 is used to check the CO in the exhaust and the speed of the vehicle respectively.

LDR is used to verify the light intensity of the vehicle. CAN bus is a vehicle bus standard designed to allow microcontrollers and devices to communicate with each other within a vehicle without a host computer. CAN bus is a message-based protocol, designed specifically for automotive applications but now also used in other areas such as aerospace, industrial automation and medical equipment. These sensors used in this project are very reliable and easily available. The interfacing of this sensor to the central unit helps the user to interact with the vehicle more easily.

Personal computer                          USB-CAN smart card		           CAN bus CAN nodes

CAN node1

MC9S12DP256

Monitoring system application

PDI

USBD12

Port A

CAN node2

DC-DC converter

DC-DC converter

MS CAN module

USB root hub

USB

port

CAN interface

photo coupler

CAN transceiver

CAN interface

photo coupler

CAN transceiver

Power module

CAN node3

Figure 3.1 Block Diagram Of Existing Method

Digital control of the vehicle is an important criterion of modern technology. With the rapid development of embedded technology, high performance embedded processor is penetrated into the auto industry, which is low cost, high reliability and other features to meet the needs of the modern automobile industry. The proposed high speed CAN bus system solves the problem of automotive system applications, also has a certain practical value and significance .With ARM as the main controller and it makes full use of the high performance of ARM, high speed reduction of CAN bus communication control.

The ARM based data acquisition system that uses ADC to bring all control data from analog to digital format. The communication module used in this project is embedded networking by CAN which has efficient data transfer. The CAN Protocol is necessary for the different control systems (and their sensors) to exchange information.

This is usually done by discrete interconnection of the different systems (i.e. point to point wiring). The requirement for information exchange has then grown to such an extent that a cable network with a length of up to several miles and many connectors was required. The benefits of CAN is effectively implemented in vehicle it is used for achieving automation, over other tradition schemes it will offer increase flexibility and expandability for future technology. Generally a vehicle was built with an analog driver vehicle interface for indicating various vehicle statuses like Speed, Fuel level, Engine temperature etc.

The CAN is provide a high speed and the capacity is high it is capable for handling a large number of parameter with more efficiently .The parameters like engine temperature if the temperature increase above the 600°C the automatically cooling system apply due to this temperature is not exceed, speed measure using RPM sensor if revolution increase up to 70 per minute controller act and to avoid the maximum revolution and to check the fuel level continuously and display in the percentage if fuel level below 20 percent the controller gives buzzer to the driver and fuel level and temperature continuously display on the LCD.

3.2 PROPOSED METHOD

Controller Area Network uses in wide variety of applications in automobiles, industries etc.., The  CAN  module  is  a  serial  interface,  useful  for  communicating  with  other MSP430 controller devices. The protocol is designed for communication within noisy environments.

Unlike a traditional network such as USB or Ethernet, CAN does not send large number of blocks of data. It provides the acknowledgement, whether the data is received or not. It will send the short messages from transmitter to receiver. If the data rate is very low, it automatically discards the data.

MSP430 is a low power consuming microcontroller which has been used in this project for higher efficiency in Battery Life Period. This controller with low supplement can carry out all data analysis and monitoring process in automotive.

CHAPTER 4

DESIGN AND IMPLEMENTATION

4.1 PROJECT DESCRIPTION

In  vehicle control system implementation using can protocol is implemented, by using  MSP430  microcontroller  interfacing can be with different  controllers  via  CAN  Protocol.  In  this  project by  using  different analog sensors that will continuously monitor the vehicle and sending the values to  the  MSP430  controller   via  ADC  interface as shown in Figure 4.1. There  are  three   analog  sensors. Analog  sensor  used  is  LM35  that  is  used  to  monitor  the  temperature  of  the vehicle.

Figure 4.1 Block Diagram Of Proposing Method

CAN  bus  is  vehicle   bus   standard  designed   to   allow   microcontrollers  and   devices  to communicate  with  each other  within  a  vehicle  without  a  host computer. CAN bus   is a  message-based  protocol,   designed   specifically   for   automotive applications  but  now  also  used  in  other  areas   such  as  aerospace,  industrial automation and  medical equipment. These sensors  used in this project are  very reliable and easily available.  The  interfacing of  this sensor to  the central  unit helps the user to interact with the vehicle more easily.

4.2 CIRCUIT OPERATION

In transmitter side, 12V supply is given to the voltage regulator and it gives the regulated voltage as 5V. It is used to run the MSP430  microcontroller. Pin 1 of the  controller   is connected  to  high  supply   voltage  in  order  to  clear  the controller. Every time, when we start it will clear the microcontroller. Colpitts oscillator is used in micro controller in order to maintain the constant frequency. A 10 kΩ  resistor is connected to pin 1in transmitter side and    it acts as a pull  up resistor. 22 µF is used in the controller to reduce the spikes in the transmitter side. Gas sensor is connected to the 4 pin of the controller ,temperature sensor is connected  to  the  1st pin   and  LDR  is connected  to the  1.1st pin  of  the controller.

In LCD, VSS pin is connected to the supply and VDD is connected to ground.  The control signal is RS pin connected to the 2.1th   pin of the controller. RW  is  connected  to  the  2.2th  pin  and  EN  is  connected  to  the 2.3th   pin  of  the  controller. The data pins D1 to D4  of the LCD connected to the 2.3th  to 2.6th  of the controller.  And 1.5th  pin is connected to the transmitter of  mcp2551 transceiver.

In  receiver  side,  similar  to  transmitter  it  has  a  microcontroller. A 12V supply is given to the voltage regulator and it gives the regulated voltage as 5V. It is used to run the  MSP430  microcontroller. Pin 1 of the controller is connected to high supply voltage in order to clear the controller. Every time, when we start it will clear the microcontroller. Colpitts oscillator is used in micro controller in order to maintain the constant frequency. A 10 kΩ  resistor is connected   to   pin   1 in    transmitter  side    and  it  acts  as  a  pull  up  resistor.

22 µF  is  used  in  the controller to reduce the spikes in the receiver side.

1.6th pin is connected to the receiver of the mcp2551 transceiver in the receiver side.  And  LCD  is  interface  with  MSP430  microcontroller  as,  VSS  pin  is connected to the supply and VDD is connected to ground. The control signals RS pin is connected to the 2.1th  pin of the controller, RW is connected to the 2.2th   pin and EN is connected to the  2.3th of the  controller. The data pin D1 to D4 of the LCD is connected to the 27th  to 30th pin of the controller.

4.3 CAN PROTOCOL

MSP430 microcontroller Embedded with CAN controller

CAN bus

CAN transceiver

Figure 4.2 CAN Architecture

The CAN Protocol was developed by BOSCH as a  multi-master, message broadcast  system  that  specifies a maximum signaling rate of 1 megabit per second (mbps). CAN is an International Standardization Organization (ISO) defined  serial  communications  bus   originally  developed  for  the   automotive  industry  to  replace   the  complex  wiring   harness  with  a  two-wire   bus.  The  specification calls for high immunity to  electrical  interference and the ability to  self-diagnose   and  repair   data   errors.   These  features   have   led   to  CAN’s  popularity in a variety of industries including  building automation,  medical  and  manufacturing.

CAN controller

CAN controller

CAN transceiver

CAN transceiver

CAN L                   CAN H	                CAN L	           CAN H

CAN BUS

CAN   Bus

Figure 4.3 Conceptual Architecture

The CANL output drives the  Low side of  the CAN differential bus. From Figure 4.3 the pin is also tied internally to receive the input comparator. The CANH output drives the high-side of the CAN differential bus. This pin is also tied internally to receive the input comparator.

4.3.1 CAN Bus Module Features

- Complies with ISO CAN Conformance Test.

- Message bit rates up to 1 Mbps.

- Conforms to CAN 2.0B Active Spec with:

  - 29-bit Identifier Fields.

  - 8-byte message length.

  - 3 Transmit Message Buffers with prioritization.

  - 2 Receive Message Buffers.

  - 6 full, 29-bit Acceptance Filters.

  - Prioritization of Acceptance Filters.

  - Multiple Receive Buffers for High Priority.

- Messages to prevent loss due to overflow.

- Advanced Error Management Features.

4.4 CAN MODULE OVERVIEW

The Controller Area Network(CAN) module is a serial interface, useful for communicating with other peripherals or microcontroller devices. This interface protocol was designed to allow communications within noisy environments. The CAN  bus  module  consists  of  a  protocol  engine  and  message  buffering  and control.  The  CAN   protocol  engine  handles  all   functions  for  receiving  and transmitting messages on the CAN bus.

Messages are transmitted by   first  loading the appropriate data registers. Status  and  errors  can  be  checked  by  reading  the  appropriate  registers. Any message  detected  on  the  CAN  bus  is  checked  for  errors  and  then  matched against filters to see if it should be received and stored in one of the two receive registers.

The CAN module supports the following frame types:

- Standard Data Frame.

- Extended Data Frame.

- Remote Frame.

- Overloaded Frame.

- Error Frame.

- Interframe Space.

4.4.1 Standard Data Frame

| S O F | 11 Bit Identifier | R T R | I D E | r0 | D L C | 0…8 Bytes Data | CRC | ACK | E O F | I F S |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 4.4 Frame Format For CAN

The various standard Data Frames are explained as follows as shown in Figure 4.4,

- SOF–The single dominant Start Of Frame (SOF) bit marks the start of a message,and is used to  synchronize  the  nodes  on  a bus  after  being idle.

- Identifier-The Standard CAN 11-bit Identifier establishes the priority of the  message. The  lower  the  binary  value,  the  higher  its  priority.

- RTR–The single Remote Transmission Request (RTR) bit is dominant when information is required from another node. All nodes receive the request, but the  identifier determines the specified    node. The  responding data  is also received by all nodes and used by any node interested. In this way, all data being used in a system is uniform.

- IDE–A   dominant  single  IDentifier   Extension  (IDE)  bit   means  that a  standard  CAN  identifier  with  no  extension  is  being  transmitted.

- r0 – Reserved bit (for possible use by future standard amendment).

- DLC–The 4-bit Data  Length Code (DLC) contains the   number of bytes of data being transmitted.

- ACK–Every node receiving an accurate message overwrites this recessive bit in the original   message with a dominate bit,    indicating an error-free message has been sent. If the receiving node detects an error and leaves this bit recessive, it discards the message and the sending node repeats the message after  rearbitration. In this  way, each  node acknowledges (ACK) the integrity of  its data.  ACK is two bits,  one is the acknowledgment bit and the second is a delimiter.

- EOF–This End-Of-Frame (EOF), 7-bit field marks the end of a CAN frame (message)  and   disables  bit-stuffing,  indicating   a  stuffing  error   when dominant. When 5 bits of the same logic level occur in succession during normal operation, a bit of the opposite logic level is stuffed into the data.

- IFS–This 7-bit Inter Frame Space (IFS) contains the time required by the controller to move a correctly received frame to its proper position in a message buffer area.

4.4.2 Remote Frame

The purpose of remote frame is used to request transmission of a corresponding DATA FRAME from a remote node. The remote frame is similar to data frame, with  two  important differences.  This type  of  message  is explicitly  marked  as  a remote frame  by a recessive  RTR bit  in the arbitration field,  and  there  will  be  no data.

4.4.3 Error Frame

The  error  frame  is  a  special message  that violates the formatting rules of a CAN message. It  is  transmitted  when  a  node detects an error in a message, and causes all  other  nodes  in  the  network  to  send  an  error  frame  as  well.  The  original transmitter then automatically retransmits the message.

4.4.4 Overload Frame

The overload frame is similar to the error frame with regard to the format, and it is transmitted by a node that becomes too busy. It is primarily used to provide for an extra delay between messages.

4.4.5 Inter Frame Space

Data frame and   remote  frame are separated  by an  interframe space. The time required  by  the  controller  to  move  a  correctly  received  frame  to  its  proper  position in a message buffer area.

4.5 CAN Bus Arbitration

A fundamental  CAN characteristic shown is the opposite logic state between  the  bus,  and  the  driver  input  and  receiver  output. Normally, a logic-high   is associated  with  a  one,  and  a logic-low  is  associated  with  a  zero  but not  so  on  a  CAN  bus. This  is  why TI  CAN  transceivers  have  the  driver input and receiver output pins passively pulled high internally, so that in the absence of any input,  the  device  automatically  defaults  to  a  recessive  bus  state on all input and output pins.

VDIFF

Dominant

Voltage Level (V)

CANH

Recessive			      Recessive

CANL

Time (t)

Figure 4.5 CAN Bus Arbitration

Bus access is event-driven and takes place randomly. If two  nodes try to occupy  the  bus  simultaneously,  access  is  implemented  with  a  nondestructive, bit-wise arbitration. Nondestructive means that the node winning arbitration just continues  on   with   the  message,   without  the   message  being   destroyed   or corrupted  by   another  node.  The   allocation  of  priority   to  messages   in  the identifier is a feature of CAN that makes it particularly attractive for use within a real-time control environment.

The  lower the binary  message  identifier number, the higher its priority. An identifier  consisting  entirely  of  zeros  is  the  highest  priority  message  on a network because it holds the bus dominant the longest as shown in Figure 4.5. Therefore,  if two nodes begin  to transmit simultaneously, the   node that sends a   last identifier bit as a zero (dominant) while the other nodes send a one (recessive) retains control of the  CAN  bus  and  goes  on  to  complete  its  message.  A  dominant  bit  always overwrites a recessive bit on a CAN bus.

C wins	        	B wins

Arbitration	           	Arbitration

Node C

Transmits

Node B

Transmits

CAN Bus

Figure 4.6 CAN Bus Arbitration in nodes

Note  that  a  transmitting  node  constantly  monitors  each  bit  of  its  own transmission. This is the reason for the transceiver configuration is in which the CANH and CANL output pins of the driver are internally tied to the receiver's input. The propagation delay of a signal in the internal loop from the driver input to the receiver output is typically used as a qualitative measure of a CAN transceiver. This propagation delay is referred to as the loop time (tloop in a TI data sheet), but takes on varied nomenclature from vendor to vendor.

It displays the CAN arbitration process that is handled automatically by a CAN    controller. Because each  node  continuously  monitors  its  own transmissions, as  node B's  recessive  bit  is  overwritten  by  node  C’s   higher priority dominant bit, B detects that the   bus state does not match the bit that    it transmitted.

Therefore,  node  B  halts  transmission  while  node  C  continues  on with its message. Another attempt to transmit the message is made by node B once the bus  is released by  node C as shown in Figure 4.6. This  functionality is part of the ISO 11898 physical  signaling  layer,  which  means  that  it  is  contained  entirely  within  the CAN controller and is completely transparent to a CAN user.

The allocation of message priority is up to a system designer, but industry groups mutually agree on the significance of certain messages. For example, a manufacturer  of motor  drives  may  specify  that  message  0010  is  a  winding current feedback signal  from a motor on a CAN network and that 0011 is the tachometer  speed.  Because  0010  has  the   lowest  binary  identifier,  messages relating to current   values always  have a higher priority on the bus than those concerned with tachometer readings.

4.6 CAN Error Management

The main advantage of CAN protocol is its error detection and error handling mechanisms. The following errors can be detected

- CRC error.

- Acknowledge error.

- Form error.

- Bit error.

- Stuff bit error.

4.6.1 CRC Error

With  the  Cyclic  Redundancy  Check (CRC), the  transmitter  calculates  special Check  bits  for  the  bit  sequence, from  the  start of a frame until the end of the data field. This CRC sequence is transmitted in the CRC field. The receiving node also  calculates  the  CRC  sequence   using  the  same  formula  and  performs   a comparison  to  the received sequence. If a  mismatch  is detected,  the CRC  error has occurred and an error frame is generated. The message is repeated.

4.6.2 Acknowledge Error

In  the   Acknowledge  field  of  a  message,   the  transmitter  checks  if   the Acknowledge slot (which was sent out as a recessive bit) contains a dominant bit.  If  not,  no  other  node  has  received  the  frame  correctly.  An  Acknowledge error has occurred, an error frame is generated and the message will have to be repeated.

4.6.3 Form Error

If a node detects a dominant bit in one of the four segments, including end of  frame,interframe space,Acknowledge delimiter or CRC delimiter,then a form error has occurred and an error frame is generated. The message is repeated.

4.6.4 Bit Error

A Bit Error occurs if a transmitter sends a dominant bit and detects a recessive bit, or if it sends a recessive bit and detects a dominant bit, when monitoring the actual bus level and comparing it to the    just transmitted  bit. In the case where the transmitter sends a recessive bit and a dominant bit is detected during the arbitration  field  and  the  Acknowledge  slot,  no  Bit  Error  is  generated  because normal arbitration is occurring.

4.6.5 Stuff Bit Error

If, between the start of frame and the CRC delimiter, six consecutive bits with the same polarity are detected,the bit stuffing rule has been violated. A Stuff Bit Error occurs and an error frame is generated. The message is repeated.

4.7 TEMPERATURE SENSOR

Figure 4.7 LM35 Temperature sensor

The  LM35 is  an integrated  circuit  sensor  that  can be used  to  measure temperature with an electrical output proportional   to the temperature (in  °C). It measures  temperature  more  accurately  than  a  using  a  thermistor.  The  sensor circuitry  is  sealed  and  not  subject  to  oxidation,  etc.  The  LM35  generates  a higher output voltage than thermocouples and may not require that the output voltage be amplified. It has an output voltage that is proportional to the Celsius temperature. The scale factor is .01V/°C.

The  LM35  does  not  require  any  external  calibration  or  trimming  and maintains  an  accuracy  of  +/-0.4°C  at  room  temperature  and  +/-  0.8°C  over  a range of 0°C to +100°C. Another important characteristic of the LM35 is that it draws  only  60µA from  its  supply  and  possesses  a  low  self-heating capability. The sensor self-heating causes    less than 0.1  °C temperature rise in still air. The sensor has a sensitivity of 10mV / °C.

4.7.1 Applications

- The  LM35  is applied easily  in  the same  way as other   integrated-circuit temperature  sensors.  Glue  or  cement  the  device  to  a  surface   and  the temperature should be within about 0.01°C of the surface temperature.

- To minimize this problem, ensure that the wiring to the LM35, as it leaves the device, is held at the same temperature as the surface of interest.

- The easiest way to do this is to cover up these wires with a bead of epoxy which will insure that the leads and wires are all at the same temperature as the surface, and that the temperature of the LM35 is not affected by the air temperature.

4.8 LDR

A  photo  resistor   or  light-dependent  resistor (LDR)or  photocell   is  a  light controlled variable resistor works on the  principle of photo conductivity. Photo conductivity  is  an  optical  phenomenon  in which  the  materials  conductivity  reduces when light is absorbed by the material. When light falls i.e., photons fall  on the device, the electrons in the valence band of the semiconductor material are excited to the conduction band.

These photons in the incident light should have energy greater than the band  gap of the  semiconductor  material to  make the   electrons jump  from  the valance band to the conduction band. Hence when light having enough energy is incident on the device more and more electrons are excited to the conduction band which results in large number of charge carriers.

4.9 LCD

LCD (Liquid Crystal Display) screen is an electronic display module and find a wide  range of applications.  A 16x2   LCD display  is  very basic  module and  is very commonly   used  in   various  devices   and   circuits.  These   modules  are preferred  over  seven  segments  and  other  multi  segment  LEDs.  The  reasons being:  LCDs   are  economical;   easily  programmable;  have   no  limitation   of displaying  special  and  even  custom   characters  (unlike   in  seven   segments), animations and so on.

A 16x2 LCD  means it can display  16 characters per  line and  there are 2 such  lines.  In  this  LCD  each  character  is  displayed  in  5x7  pixel  matrix.  This LCD has two registers, namely, Command and Data.

Figure 4.8 LCD Pin Diagram

The command register stores the command instructions given to the LCD. A  command  is   an  instruction  given  to   LCD  to  do  a   predefined  task  like initializing it, clearing its screen, setting the cursor position, controlling display etc. The data register stores the data to be displayed on the LCD. The data is the ASCII value of the character to be displayed on the LCD.

4.10 ENERGIA IDE –An Integrated Development Environment

Energia is an open-source electronics prototyping platform started by Robert Wessels in January of 2012 with the goal to bring the Wiring and Arduino framework to the Texas Instruments MSP430 based LaunchPad. The Energia IDE is cross platform and supported on Mac OS, Windows, and Linux. Energia uses the mspgcc  compiler by Peter Bigot and is based on the Wiring  and Arduino  framework. Energia includes an integrated development environment (IDE) that is based on Processing.  Energia is also a portable framework/abstraction layer that can be used in other popular IDEs. Utilize a web browser based environment with CCS Cloud at dev.ti.com. Community maintained Energia plug-ins and integrations are available for Xcode, Visual Studio, and Code Composer Studio.

The foundation of Energia and Arduino is the Wiring framework that is developed by Hernando Barragan. The framework is thoughtfully created with designers and artists in mind to encourage a community where both beginners and experts from around the world share ideas, knowledge and their collective experience. The Energia team adopts the philosophy of learning by doing and strives to make it easy to work directly with the hardware. Professional engineers, entrepreneurs, makers and students can all benefit from the ease of use Energia brings to the microcontroller.

Energia started out to bring the Wiring and Arduino framework to the Texas Instruments MSP430 LaunchPad. Texas Instruments offers a MSP430, MSP432, TM4C, C2000 and CC3200 LaunchPad. The LaunchPad is a low-cost microcontroller board that is made by Texas Instruments. The latest release of Energia supports the majority of the LaunchPad product offerings. Additional community kits from RedBearLab are also supported.

Together with Energia, LaunchPad can be used to develop interactive objects, taking inputs from a variety of switches or sensors and controlling a variety of lights, motors and other physical outputs. LaunchPad projects can be stand-alone (only run on the Target Board, i.e. your LaunchPad) or they can communicate with software running on your computer (Host PC). You can also add wireless modules to enable communication over various types of RF including Wi-Fi, NFC, Bluetooth, Zigbee, cellular and more.

4.11 SYSTEM IMPLEMENTATION

Initially, the supply from the transformer is given to the microcontroller and the relay circuit. The 230V from the transformer is converted to 5V using rectifier and voltage regulator, this 5V given to the controller. The voltage regulator 7805 is used to give 5V constant DC supply. The capacitor of about 1000 µF is used as a filter. The relay circuit is used to isolate low power circuit from high power circuit. The diode IN4007 is used in relay circuit to protect controller from back emf.

There are master and slave modules in this vehicle control system implementation. In master module, sensor has been interfaced. Analog sensors like Temperature sensor and Rain water sensor which are connected to ADC ports, where the Analog values are converted into digital. MSP430G2553 has a 10 bit ADC, so the maximum precision is 1023 as digital value. Digital sensor like Fuel level sensor is used.

These values from different sensors are transmitted through CAN frames using CAN transceiver MCP2551 which supports 1Mbps operation. One CAN transceiver at the transmitter end, which is on Master controller and other CAN transceiver at the receiver end, which is on Slave controller. These CAN transceivers are interconnected by using CAN bus.

In slave module, the MSP430G2553 controller is interfaced with MAX232 which is a dual transmitter / dual receiver that converts signals from a TIA-232 (RS-232) serial port to signals suitable for use in TTL-compatible digital logic circuits. It is an alternate serial port output of the sensor values.

CAN converts the data’s from sensors into frames and transmit through a wired channel which is CAN bus. The serial monitor in ENERGIA can display the digital values of the sensors, provided COM port of the USB and device should be similar, Baud rate of the controller terminal should be 9600Hz and tool selection should be made. Through receiver which is slave module the data’s are displayed in serial monitor.

CHAPTER 5

RESULT AND DISCUSSION

The vehicle control system implementation using CAN protocol with MCP430G2553 full hardware setup is shown below Figure 5.1 without power supply (i.e. power OFF condition).

Figure 5.1 Complete Hardware Setup (power OFF condition)

The complete hardware setup of the vehicle control system is shown in the Figure 5.2 with power supply (i.e. power ON condition). The vehicle can be monitored by using the sensors (Temperature sensor, Fuel level sensor, Rain water sensor).

Figure 5.2 Complete Hardware Setup (power ON condition)

The Figure 5.3 shown in below is the serial monitor output of the Energia IDE which displays the sensor values and its responses using can transceivers in the rate of 1 Mbps.

Figure.5.3 Serial Monitor Output Screen Shot.

CHAPTER 6

CONCLUSION

Project on Vehicle Control System Implementation Using Can Protocol describes   about   the   implementation   of   CAN   bus   in automobile industries. Instead of using multiple wires, two wires are used. Thus the requirement of wires and complexity are reduced. Because of wide usage of vehicles and rapid development of embedded technology, CAN protocol is used for data transmission and it is known for its low cost, high reliability and better error  handling  mechanism.  Vehicle  conditions  are  monitored  by  CO  sensor, LM35 for temperature measurement, 12V battery and LDR to measure the light intensity. The main advantage of using this CAN protocol is its high speed data rate. The programming for the microcontroller interfacing with CAN protocol is verified  using  a  general  purpose  board.  The   temperature  of  engine,  battery voltage, presence of head light, and CO level in the exhaust are transferred from engine side to dashboard side via CAN protocol and these readings are displayed through LCD on the dashboard.

APPENDIX

A.1 MSP430G2553

A.1.1 MSP430G2553 PIN DIAGRAM:

Figure A.1 MSP430G2553 Pin Diagram

A.1.2 FEATURES OF MSP430G2:

• Low Supply-Voltage Range: 1.8 V to 3.6 V

• Universal Serial Communication Interface (USCI)

– Enhanced UART Supporting Auto Baud rate Detection (LIN)

– IrDA Encoder and Decoder

– Synchronous SPI

• Ultra-Low Power Consumption

– Active Mode: 230 µA at 1 MHz, 2.2 V

– Standby Mode: 0.5 µA

– Off Mode (RAM Retention): 0.1 µA

• Five Power-Saving Modes

• Ultra-Fast Wake-Up from Standby Mode in C™ Less than 1 µs

• On-Chip Comparator for Analog Signal Compare Function or Slope Analog-to-Digital

• 16-Bit RISC Architecture, 62.5-ns Instruction (A/D) Conversion Cycle Time

• 10-Bit 200-ksps Analog-to-Digital (A/D)

• Basic Clock Module Configurations Converter with Internal Reference, Sample

– Internal Frequencies up to 16 MHz With and-Hold and Auto scan Four Calibrated Frequency

– Internal Very-Low-Power Low-Frequency

– 32-kHz Crystal

– External Digital Clock Source Fuse

• Brownout Detector

• Serial Onboard Programming, (LF) Oscillator No External Programming Voltage Needed, Programmable Code Protection by Security

• Two 16-Bit Timer A with Three

• On-Chip Emulation Logic with Spy-Bi-Wire Capture/Compare Registers Interface

• Up to 24 Capacitive-Touch enabled I/O Pins

A.1.3 PIN CONFIGURATION MSP430G2553 LAUNCHPAD:

Figure A.2 MSP430G2553 Pin Configuration

A.2 APPLICATIONS OF MSP-EXP430G2553

The MSP430 can be used for low powered embedded devices. The current drawn in idle mode can be less than 1 µA. The top CPU speed is 25 MHz. It can be throttled back for lower power consumption. The MSP430 also uses six different low-power modes, which can disable unneeded clocks and CPU. Additionally, the MSP430 is capable of wake-up times below 1µs, allowing the microcontroller to stay in sleep mode longer, minimizing its average current consumption.

The device comes in a variety of configurations featuring the usual peripherals:internal oscillator, timer  including  PWM, watchdog, USART,  SPI, I²C, 10/12/14/16/24-bit ADCs and brown out reset circuitry. Some less usual peripheral options include comparators (that can be used with the timers to do simple ADC), on-chip op-amps for signal conditioning, 12-bit DAC, LCD driver, hardware multiplier, USB and DMA for ADC results. Apart from some older  EPROM (MSP430E3xx) and high volume mask ROM (MSP430Cxxx) versions, all of the devices are in-system programmable via JTAG (full four-wire or Spy-Bi-Wire) or a built in bootstrap loader (BSL) using UART such as RS232, or USB on devices with USB support.

However, limitations that preclude its use in more complex embedded systems. The MSP430 does not have an external memory bus, so it is limited to on-chip memory (up to 512 KB flash memory and 66 KB RAM) which may be too small for applications that require large buffers or data tables. Also, although it has a DMA controller, it is very difficult to use it to move data off the chip due to a lack of a DMA output strobe.

A.3 VOLTAGE REGULATOR

A voltage stabilizer is any device that keeps the voltage of a circuit at a specified level. There are many different types of voltage stabilizers but integrated circuit (IC) voltage stabilizers are among the most common. You will frequently need a voltage stabilizer for components that require regulated power. You can demonstrate the use of a voltage stabilizer in a circuit with a few components from an electronics parts store.

Figure A.3 7805 Voltage Regulator

Identify the parts of the voltage regulator. Place the voltage regulator so that you can read the printing on it. The digits "78" indicate a positive voltage regulator and the digits "05" indicate a 5-volt regulator. For a positive voltage regulator like a 7805, the left lead is the input, the middle lead is the ground and the right lead is the output. Mount the voltage regulator on the mounting board. Each of the voltage regulator's three leads should be inserted into a different hole in the mounting board so that the three holes are in the same column but different rows. Mount the light bulb on the mounting board. Insert the lead for the light bulb's positive terminal into a hole on the same row as the voltage regulator's output lead. Insert the light bulb's negative lead into a hole on the same row as the voltage regulator's ground lead.

Insert the positive lead of the battery holder into a hole on the same row as the voltage regulator's input. Insert the negative lead of the battery holder into a hole on the same row as the voltage regulator's ground and the light bulb's negative lead. Place the battery in the battery holder. The light bulb now receives a stable voltage of 5 volts even though the power supply is a 9-volt battery. This type of voltage regulator will dump the excess voltage as heat.

Table A.1 7805 Pin Description

| PIN No. | PIN | DESCRIPTION |
| --- | --- | --- |
| 1 | INPUT | In this pin of the IC positive unregulated voltage is given in regulation. |
| 2 | GROUND | In this pin where the ground is given. This pin is neutral for equally the input and output. |
| 3 | OUTPUT | The output of the regulated 5V volt is taken out at this pin of the IC regulator. |

A.4 ARCHITECTURE OF MSP430G2553

Figure A.4 Architecture of MSP430G2553

A.5 CAN TRANSCEIVER

A.5.1 Features

- Supports 1 Mb/s operation.

- Implements ISO-11898 standard physical layer.

- Suitable for 12V and 24V systems.

- Externally-controlled slope for reduced RFI emissions.

- Detection of ground fault (permanent Dominant) on TXD input.

- Power-on Reset and voltage brown-out protection.

- An unpowered node or brown-out event will not disturb the CAN bus.

- Low current standby operation.

- Protection  against  damage  due   to  short-circuit  conditions  (positive  or negative battery voltage).

- Protection against high-voltage transients.

- Automatic thermal shutdown protection.

- Up to 112 nodes can be connected.

- High-noise immunity due to differential bus implementation.

- Temperature ranges:

1. Industrial (I): -40°C to +85°C

2. Extended (E): -40°C to +125°C

Figure A.5 Pin Diagram For MCP2551

A.5.2 CAN Transceiver Overview

The  MCP2551  is  a  high-speed  CAN,fault-tolerant  device  that  serves  as  the interface   between  a  CAN  protocol controller  and  the physical  bus. The MCP2551  device  provides  differential  transmit  and  receive  capability  for  the CAN protocol controller, and  is fully compatible with the   ISO-11898 standard, including 24V requirements. It will operate at speeds of up to 1 Mb/s. Each node in a CAN system must have a device to convert the digital signals generated by a  CAN  controller  to  signals  suitable  for  transmission   over  the  bus  cabling (differential output). It also provides a buffer between the CAN controller and the high-voltage spikes that can be generated on the CAN bus by outside sources (EMI, ESD, electrical transients, etc.).

A.5.3 CAN Transceiver Block Diagram

Figure A.6 Block Diagram For CAN Transceiver

The CAN bus has two states: Dominant and Recessive. A Dominant state occurs when  the  differential  voltage  between CANH and CANL is  greater than  a  defined  voltage (e.g., 1.2V). A  Recessive  state  occurs  when  the  differential voltage  is   less  than  a defined voltage (typically  0V). The  Dominant and Recessive   states  correspond to the Low and High state of the TXD input pin, respectively.  However,  a  Dominant  state  initiated  by  another  CAN  node  will override a Recessive state on the CAN bus.

The RXD output pin reflects the differential  bus voltage between  CANH and CANL. The  Low  and  High  states  of  the  RXD  output pin correspond to the Dominant and Recessive states of the CAN bus, respectively.

A.6 OPERATING MODES

The RS pin allows three modes of operation to be selected:

- High-Speed.

- Slope-Control.

- Standby.

When in High-Speed or Slope-Control   mode, the drivers  for the CANH and CANL signals are internally regulated to provide controlled symmetry in order to minimize EMI emissions. Additionally,  the slope of the signal transitions on CANH and CANL can be controlled with   a resistor connected  from pin 8 (RS) to ground. The slope   must be proportional to the current output     at RS, which will further reduce EMI emissions.

High-Speed:High-Speed mode is selected by connecting the RS pin to VSS. In this mode, the transmitter  output  drivers  have  fast  output  rise  and  fall  times  to  support  high speed CAN bus rates.

Slope-Control:Slope-Control mode further reduces EMI by limiting the rise and fall times of  CANH and CANL. The slope, or slew rate(SR), is controlled by connecting an external resistor (REXT) between RS and VOL (usually ground). The slope is proportional to the current output at the RS pin. Since the current is primarily determined by the slope-control resistance value REXT,a certain slew rate is achieved by applying a specific resistance.

Standby Mode:The device may be placed in Standby or SLEEP mode by applying a high-level to the RS pin. In SLEEP mode, the transmitter is switched off and the  receiver  operates at a lower current. The receive pin on the controller side (RXD) is still functional, but will operate at a slower rate. The attached  microcontroller can monitor  RXD  for  CAN  bus   activity  and  place  the  transceiver  into   normal  operation via the RS pin at higher bus rates, the first CAN message may be lost.

A.6 PIN DETAILS

Transmitter Data Input (TXD):TXD is a TTL-compatible input pin. The data on this pin is driven out on the CANH  and  CANL   differential  output  pins.   It  is  usually   connected  to  the transmitter  data  output  of  the  CAN  controller  device.   When  TXD  is  Low, CANH and CANL are in the Dominant state. When TXD is High, CANH and CANL are in the Recessive state, provided that another CAN node is not driving the  CAN  bus  with  a  Dominant  state.  TXD  has  an   internal  pull-up  resistor (nominal 25 kΩ to VDD).

Ground Supply (VSS), Supply Voltage (VDD):Ground supply pin, Positive supply voltage pin.

Receiver Data Output (RXD):RXD is a CMOS-compatible output that drives High or Low depending on the differential signals on the CANH and CANL pins and is usually connected to the  receiver data  input of  the CAN controller device. RXD    is  High when  the CAN bus is Recessive and Low in the Dominant state.

Reference Voltage (VREF):Reference Voltage Output (defined as VDD/2).

CAN Low (CANL):The CANL output drives the  Low side of  the CAN differential bus. This  pin  is  also tied  internally to  receive the input comparator.

CAN High (CANH):The CANH output drives the high-side of the CAN differential bus. This  pin  is  also  tied  internally to receive the input comparator.

Table no A.2 Pin Details Of CAN Transceiver

| PIN NUMBER | PIN NAME | PIN FUNCTION |
| --- | --- | --- |
| 1 | TXD | Transmitter data input |
| 2 | VSS | Ground |
| 3 | VDD | Supply Voltage |
| 4 | RXD | Receive data output |
| 5 | VREF | Reference output voltage |
| 6 | CANH | CAN High level voltage I/O |
| 7 | CANL | CAN Low level voltage I/O |
| 8 | RS | Slope-control output |

REFERENCES

- Benjamin C Kuo.M (2003), Farid Golnaraghi, Automatic Control       systems, 8th edition, John Wiley & sons. Inc.

- Gmbh.B (1991), “CAN specifications” Volume 1 Version 2.0.

- ISO (1993). Road Vehicles: Interchange of Digital Information: “Controller Area Network (CAN) for High Speed Communication”.

- Kumar.M, Verma.A, and Srividya.A (2009), Response-Time “Modeling of Controller Area Network (CAN). Distributed Computing and Networking, Lecture Notes in Computer Science Volume 5408.

- Li.M (2009), Design of Embedded Remote Temperature Monitoring System based on Advanced RISC Machine.

- Pazul (1999), “Controller Area Network (CAN) Basics”, Microchip technology Inc.

- Prodanov.W, Valle.M, and Buzas.R (2009), “A controller area network bus transceiver behavioral model for network design and simulation”.

- Tindell .K, Burns.A, and Wellings.A.J (2005), “Calculating controller area network (CAN) message response times”.

- Vijayalakshmi.S (2013), “Vehicle control system implementation Using CAN protocol”. IJAREEIE, Volume 2.

- Wilfried Voss (2005-2008), “A comprehensive guide to controller area network”, Copperhill Media Corporation, 2005-2008.